IMPROVEMENT IN ROAD SIGNAGES ON NATIONAL HIGHWAY IN INDIA USING ARTIFICIAL INTELLIGENCE (AI) BASED SURVEYS

Name of the stretch -

4 laning of Km. 34.1 (Farukhnagar) to Km. 80 (Jadcherla) and (II) Improvement, Operation and Maintenance of Km. 22.3 (Thondupalli) to Km. 34.1 (Farukhnagar) on NH-7 in the state of Andhra Pradesh under North-South Corridor (NHDP Phase-II) on Build, Operate and Transfer (BOT) basis–NS2/BOT/AP-3

Unique Project Code (UPC) 			- N/02010/01001/TS

State 							- Telangana

Regional Office (RO)				- Hyderabad

Project Implementation Unit (PIU)  	- Mancherial

Table of Contents

Executive Summary	3

1.	Introduction	5

1.1.	Brief description about the MoU	5

1.2.	Objectives of Gap Study	6

1.3.	Scope of Services	6

1.4.	Salient features of the instant National Highway Project	7

2.	Methodology	9

2.1.	Data Collection	9

2.2.	Broad Methodology- Artificial Intelligence based solution	9

3.	Inventory of Road signs	10

3.1.	Road Signage Inventory (as provided by NHAI)	10

3.2.	Road Signage Inventory based on AI Survey Data (conducted by IIITD)	10

3.3.	Road signage requirement as per Road Safety Auditor recommendation	10

4.	Results of Gap study	13

4.1.	Gap study based on NHAI data	13

4.2.	Gap study based on Road safety auditor recommendation	14

Annexure A	16

Annexure B	18

Annexure C	61

Annexure D	124

# Executive Summary

National Highways Authority of India (NHAI), under Ministry of Road Transport and Highways (MoRTH), Government of India has signed a Memorandum of Understanding (MoU) with Indraprastha Institute of Information Technology, Delhi (IIITD) to utilize AI based solutions for carrying out Gap Study w.r.t availability and broad condition of road sign on around 25,000 km of selected National Highways (provided by NHAI). The gap study shall be carried out by assessing the difference between the AI based survey findings and the road signage inventory as per NHAI record. The other part of the gap study shall  cover the additional requirement of road signages as per the recommendations of certified Road Safety Auditor (RSA) based on latest codal provisions relevant to high-speed corridors.

The MoU was signed on 19-07-2024 and project completion period was kept as twelve (12) months. The date of commencement of this project was 15-10-2024.

This report summarizes the findings of a gap study conducted on National Highway project 4 laning of Km. 34.1 (Farukhnagar) to Km. 80 (Jadcherla) and (II) Improvement, Operation and Maintenance of Km. 22.3 (Thondupalli) to Km. 34.1 (Farukhnagar) on NH-7 in the state of Andhra Pradesh under North-South Corridor (NHDP Phase-II) on Build, Operate and Transfer (BOT) basis–NS2/BOT/AP-3.The linear length of project stretch is 58.085 Km and it includes “Four/SixLane with divided carriageway with Service Road” configuration and the type of pavement is Flexible/Rigid. The construction work of the project was completed on/ date of COD was_. The project is currently under the O/DLP stage.

The survey on this project stretch was carried out on 02 April 2026 by the team of IIIT Delhi. As per survey findings which is based on Artificial Intelligence (AI), there are 534 nos. of roads signs including Chevron, Hazard, Cautionary Warning, Prohibitory Mandatory  Informatory Signs, on this project. However, as per NHAI record/ Approved Road Signage Plan of Contract Agreement, the number of road signages on this project are 0 nos. (data as provided by NHAI). Therefore, an excess of 534 nos. sign boards have been observed vis-à-vis NHAI record. This variance in the count of signages in the contract document and existing signages taken out by AI survey report is not available in NHAI record. This finding will help NHAI in better inventorisation of the existing Road Signage on this project.

Secondly, this project has also carried out gap study based on the recommendation of certified road safety auditor (RSA). As per RSA recommendation, the number of Total road signs required on this project are 681 nos. Accordingly, there is an additional requirement of 147 nos. of signages over and above 534 nos. as observed during the survey.

The following table presents the summary of Gap study report:

### INSERT_EXECUTIVE_SUMMARY_TABLE ###

# Introduction

## Brief description about the MoU

A Memorandum of Understanding (MoU) has been signed between the National Highways Authority of India (NHAI) and Indraprastha Institute of Information Technology, Delhi (IIIT Delhi) for carrying out Gap Study w.r.t availability and broad condition of road sign on around 25,000 km National Highways in India.

The project duration as per the MoU is twelve (12) months and the date of commencement of work is 27.09.2024.  The tentative length of roads to be covered under the aforementioned study shall be 25,000 km. The list of stretches included in the project are from different states which is divided into 05 zones (Zone A to E).

| Zone | States |
| --- | --- |
| Zone A | Jammu Kashmir, Punjab, Himachal Pradesh, Uttarakhand , Haryana , Chandigarh and Delhi |
| Zone B | Gujrat and Rajasthan |
| Zone C | Maharashtra and Madhya Pradesh |
| Zone D | Andhra Pradesh , Tamil Nadu , Telangana , Karnataka , Kerala , Chhattisgarh |
| Zone E | Assam , Bihar , Meghalaya , Jharkhand , Odisha , Uttar Pradesh and West Bengal |

## Objectives of Gap Study

Adequate availability of the road signs on the roads plays a significant role in the road safety. NHAI intends to enhance the road safety for all road users by embracing innovation and adopting advanced technologies.

Artificial Intelligence (Al) has emerged as a powerful tool for automating tasks and improving data analysis capabilities. By harnessing the potential of Al and Geographic Information Systems (GIS), NHAI can revolutionize its approach w.r.t road signs inspection.

Therefore, NHAI in collaboration with IIIT Delhi intends to utilize Al based solutions for Improvement in the availability of road signs on National Highways in India.

## Scope of Services

The scope of work is to carry out Gap studies w.r.t the availability and broad condition of road signs on around 25,000 km of National Highways in India.  The services includes following:

- The gap study shall be carried out by assessing the difference between the survey findings and the requirements of road signs as per signage plan of the respective Contract Agreement.

- Gap study based on updated / latest Codal provisions relevant to high-speed corridors to cater for enhanced safety requirements. For this purpose, IIITD shall engage a certified Road Safety Auditor (RSA) for assisting in the gap study and preparation of report.

- IIITD shall carry out the surveys for collecting imagery and other ancillary data related to availability and condition of road signages on selected National Highways stretches in India as provided by NHAI.

- The data collected through surveys shall be processed through deployment of adequately capable Artificial Intelligence (AI) for accurate identification and classification of road sign types.

## Salient features of the instant National Highway Project

- The salient features of the instant National Highway Project are mentioned as under:

| S. No. | Description/ Particular | Details |
| --- | --- | --- |
| 1 | Name of the project Highway | 4 laning of Km. 34.1 (Farukhnagar) to Km. 80 (Jadcherla) and (II) Improvement, Operation and Maintenance of Km. 22.3 (Thondupalli) to Km. 34.1 (Farukhnagar) on NH-7 in the state of Andhra Pradesh under North-South Corridor (NHDP Phase-II) on Build, Operate and Transfer (BOT) basis–NS2/BOT/AP-3 |
| 2 | Unique Project Code (UPC) | N/02010/01001/TS |
| 3 | NHAI Regional Office | Hyderabad |
| 4 | NHAI Project Implementation Unit (PIU) | Mancherial |
| 5 | Details of Project Construction; |  |
|  | (a) Date of Completion/ COD |  |
|  | (b) DLP Completion Date |  |
| 6 | Details of OM Agency (if any); |  |
|  | (a) OM Contract Duration |  |

- Key Features of the Project:

| S. No. | Description/ Particular | Details |
| --- | --- | --- |
| 1 | Pavement Type Lane Configuration |  |
| 2 | Length of Main Carriageway |  |
| 3 | Length of Service/ Slip Roads |  |
| 4 | Major Junctions |  |
| 5 | Minor Junctions |  |
| 6 | Bridges (Major/Minor) |  |
| 7 | ROB/RUB/Flyovers |  |
| 8 | Underpasses (VUP/PUP/CUP) |  |
| 9 | Culverts |  |
| 10 | Toll Plaza |  |

4 laning of Km. 34.1 (Farukhnagar) to Km. 80 (Jadcherla) and (II) Improvement, Operation and Maintenance of Km. 22.3 (Thondupalli) to Km. 34.1 (Farukhnagar) on NH-7 in the state of Andhra Pradesh under North-South Corridor (NHDP Phase-II) on Build, Operate and Transfer (BOT) basis–NS2/BOT/AP-3 – Zone C

- Index map:

Index map showing the start point and end point of the said project.

None

- Total Road segments in this project

Main Carraige Way LHS Surveyed - 1

Main Carraige Way RHS Surveyed - 1

### INSERT_ROAD_TABLE ###

# Methodology

## Data Collection

The research methodology used is a combination of data collection and processing the same through Artificial Intelligence (AI) based solutions. The data collection involves the use of technology like integrated device which is mounted on the vehicle and the survey is done with expert and driver in the vehicle. The start point is marked and the Main Carriageway (MCW) is covered on both LHS  RHS. The survey further includes covering all the service roads and intersections of the same stretch on vehicles to collect data remotely. The data which is collected is uploaded for the further processing through AI models.

The information from respective PIUs were captured through an automated form to make the process of Data collection Streamline.

## Broad Methodology- Artificial Intelligence based solution

AI models are employed to process the collected data, identify patterns, and generate insights into road signages as seen in the video captured. The chainage, name of the road, latitude  longitude are marked simultaneously.

# Inventory of Road signs

## Road Signage Inventory (as provided by NHAI)

The inventory of road signs as provided by NHAI with broad categorisation is presented in the table below. The detailed inventory is placed at Annexure A.

### INSERT_INVENTORY_SECTION_ONE ###

## Road Signage Inventory based on AI Survey Data (conducted by IIITD)

The inventory of road signs based on AI Survey Data with broad categorisation is presented in the table below. The detailed inventory is placed at Annexure B.

### INSERT_INVENTORY_SECTION_TWO ###

## Road signage requirement as per Road Safety Auditor recommendation

The inventory of road signs as per Road Safety Auditor recommendation is presented in the table below. The detailed inventory is placed at Annexure C.

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |
| RSA Name: | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |
| Project Name: | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |
| State/RO: | Telangana |  | PIU: | Mancherial |  |
| Start Chainage : | 0 m |  | End Chainage : | 42000 m |  |
| Length (Km) : | 42 kms |  |  |  |  |
| Summary of Additional Signages |  |  |  |  |  |
| Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Decription | No. of Signs |  |  |  |
|  |  | Avenue/Left | Median/Right | Overhead | Total Signages |

# Results of Gap study

## Gap study based on NHAI data

### RESULT_GAP_STUDY_ONE ###

## Gap study based on Road safety auditor recommendation

Following table represents the gap analysis of road signs between NHAI record , AI survey data and Road safety auditor recommendation with different category of signage. The detailed inventory is placed at Annexure D.

### RESULT_GAP_STUDY_TWO_TABLE ###

Following graph shows the difference between NHAI record , AI based survey and Road safety auditor recommendation with different category of signage in a bar chart .

### RESULT_GAP_STUDY_TWO_GRAPH_ONE ###

Following graph shows the difference between NHAI record , AI based survey and Road safety auditor recommendationin a bar chart format

### RESULT_GAP_STUDY_TWO_GRAPH_TWO ###

# Annexure A

### ANX_A ###

# Annexure B

### ANX_B ###

# Annexure C

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW LHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 0+145 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 0+225 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |
| 0+300 | Prohibitory Mandatory Signs | 14.35 | Height Limit |  | Avenue |  |  |  |
| 0+430 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 0+495 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 0+560 | Cautionary Warning Signs | 15.10 | Side Road Left |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW LHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 0+005 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |
| 0+025 | Prohibitory Mandatory Signs | 14.35 | Height Limit |  | Avenue |  |  |  |
| 0+235 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 1+165 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |
| 7+990 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 8+310 | Hazard | 15.80 | Two Way Object Hazard Marker |  | Avenue | Yes |  |  |
| 8+310 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |
| 9+195 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW LHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 9+290 | Hazard | 15.80 | Two Way Object Hazard Marker |  | Avenue | Yes |  |  |
| 9+290 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |
| 9+875 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 12+680 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 12+770 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 13+195 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 13+825 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 14+820 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW LHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 14+900 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 17+535 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 17+605 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 21+490 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |
| 21+490 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Median |  |  |  |
| 21+872 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW LHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 23+915 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 23+975 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 24+050 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 24+855 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Median |  |  |  |
| 25+055 | Hazard | 15.78 | Object Hazard Left |  | Avenue | Yes |  |  |
| 26+095 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 26+113 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW LHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 26+195 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 26+588 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 29+205 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 29+225 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 29+268 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 29+555 | Hazard | 15.78 | Object Hazard Left |  | Avenue | Yes |  |  |
| 30+445 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Median |  |  |  |
| 31+190 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW LHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 36+185 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Median |  |  |  |
| 36+445 | Cautionary Warning Signs | 15.22 | Merging Road Ahead From Left |  | Avenue |  |  |  |
| 36+960 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 39+890 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 39+955 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 40+028 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 41+015 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Median |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 41+835 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 40+955 | Hazard | 15.79 | Object Hazard Right |  | Median | Yes |  |  |
| 40+955 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Median |  |  |  |
| 40+200 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 40+060 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 40+030 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Median |  |  |  |
| 40+000 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 39+930 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 39+880 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 38+150 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 37+800 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 37+720 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 37+620 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 37+045 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 36+850 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 36+180 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 36+055 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Median |  |  |  |
| 35+070 | Hazard | 15.78 | Object Hazard Left |  | Avenue | Yes |  |  |
| 34+915 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 34+750 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 34+670 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 34+620 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Median |  |  |  |
| 34+610 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 34+560 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 34+520 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 34+500 | Cautionary Warning Signs | 15.02 | Right Hand Curve |  | Avenue |  |  |  |
| 34+050 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 33+830 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 33+790 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 33+690 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 31+700 | Hazard | 15.78 | Object Hazard Left |  | Avenue | Yes |  |  |
| 31+230 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 30+430 | Hazard | 15.80 | Two Way Object Hazard Marker |  | Avenue | Yes |  |  |
| 30+430 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |
| 30+380 | Hazard | 15.78 | Object Hazard Left |  | Avenue | Yes |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 30+380 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Median |  |  |  |
| 29+620 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 29+520 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 29+450 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 29+400 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Median |  |  |  |
| 29+330 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 29+200 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 27+930 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 27+750 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 27+650 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 27+630 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 27+560 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 27+445 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 27+340 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 26+820 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 26+690 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 26+400 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 26+350 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 26+330 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 26+210 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Median |  |  |  |
| 26+140 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 26+050 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 25+650 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 25+600 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 25+570 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 25+490 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 25+165 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 24+805 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |
| 24+790 | Hazard | 15.78 | Object Hazard Left |  | Avenue | Yes |  |  |
| 24+785 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Median |  |  |  |
| 24+610 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 24+365 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 24+220 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 24+030 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 24+000 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Median |  |  |  |
| 23+970 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 23+945 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 23+880 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 22+250 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 22+120 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 22+100 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 22+040 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 21+800 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 21+640 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 21+485 | Hazard | 15.80 | Two Way Object Hazard Marker |  | Median | Yes |  |  |
| 21+485 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Median |  |  |  |
| 21+485 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |
| 21+470 | Cautionary Warning Signs | 15.60 | Barrier |  | Median |  |  |  |
| 21+400 | Informator y Signs | 16.09 | Weigh Bridge |  | Avenue |  |  |  |
| 21+390 | Informator y Signs | 17.05 | Toilet |  | Avenue |  |  |  |
| 21+300 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 21+250 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 21+240 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 21+150 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 19+900 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 19+750 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 18+500 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 18+380 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 18+330 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 18+280 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 18+075 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 17+720 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 17+650 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 17+570 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Median |  |  |  |
| 17+530 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 17+450 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 16+800 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 16+720 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 16+700 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 16+620 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 15+070 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 14+910 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 14+870 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Median |  |  |  |
| 14+850 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 14+775 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 14+720 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 14+000 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 13+840 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 13+820 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 13+740 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 13+550 | Cautionary Warning Signs | 15.01 | Left Hand Curve |  | Avenue | Yes |  |  |
| 13+170 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 12+950 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 12+840 | Cautionary Warning Signs | 15.22 | Merging Road Ahead From Left |  | Avenue |  |  |  |
| 12+810 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 12+730 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 12+710 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Median |  |  |  |
| 12+680 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 12+600 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 11+250 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 11+200 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 11+115 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 10+995 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 10+950 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 10+900 | Cautionary Warning Signs | 15.01 | Left Hand Curve |  | Median |  |  |  |
| 10+100 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 10+000 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 9+950 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 9+740 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 9+190 | Hazard | 15.78 | Object Hazard Left |  | Avenue | Yes |  |  |
| 9+070 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 8+270 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 7+250 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 6+700 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 6+630 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 6+600 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |
| 6+500 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 6+410 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 4+920 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 4+850 | Cautionary Warning Signs | 15.11 | Y - Intersection |  | Avenue |  |  |  |
| 4+810 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Median |  |  |  |
| 4+750 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Avenue |  |  |  |
| 4+590 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 4+105 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 2+500 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 2+310 | Hazard | 15.78 | Object Hazard Left |  | Avenue |  |  |  |
| 1+500 | Cautionary Warning Signs | 15.01 | Left Hand Curve |  | Median |  |  |  |
| 1+400 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 1+300 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 1+250 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 1+100 | Prohibitory Mandatory Signs | 14.29 | No Standing |  | Avenue |  |  |  |
| 0+145 | Hazard | 15.79 | Object Hazard Right |  | Median |  |  |  |
| 0+135 | Cautionary Warning Signs | 15.01 | Left Hand Curve |  | Median |  |  |  |
| 0+650 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Avenue |  |  |  |
| 0+600 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 0+475 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Avenue |  |  |  |
| 0+390 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Median |  |  |  |
| 0+250 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Avenue |  |  |  |
| 0+200 | Prohibitory Mandatory Signs | 14.25 | U - Turn Prohibited |  | Avenue |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  | 42 kms |  | Carriageway : | MCW RHS |  |  |  |
| Chainage | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Avenue/Median/ Overhead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 0+100 | Cautionary Warning Signs | 15.32 | Gap In Median |  | Median |  |  |  |
| 0+080 | Cautionary Warning Signs | 15.73 | U-Turn Ahead |  | Median |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 0+275 | SRL 1 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 0+285 | SRL 1 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 0+450 | SRL 1 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 0+540 | SRL 1 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 0+890 | SRL 1 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 0+900 | SRL 1 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 1+090 | SRL 1 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 1+100 | SRL 1 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Left |  |  |  |
| 1+150 | SRL 1 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Left |  |  |  |
| 1+200 | SRL 1 | Prohibitory Mandatory Signs | 14.25 | U-Turn Prohibited |  | Left |  |  |  |
| 1+230 | SRL 1 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 1+310 | SRL 1 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 1+730 | SRL 1 | Cautionary Warning Signs | 15.10 | Side Road Left |  | Left |  |  |  |
| 1+800 | SRL 1 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 1+820 | SRL 1 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left |  |  |  |
| 2+790 | SRL 1 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |
| 2+810 | SRL 1 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left |  |  |  |
| 3+320 | SRL 1 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Left |  |  |  |
| 3+350 | SRL 1 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 3+400 | SRL 1 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 3+410 | SRL 1 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 5+100 | SRL 2 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 5+200 | SRL 2 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Left |  |  |  |
| 5+390 | SRL 2 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 5+400 | SRL 2 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left |  |  |  |
| 6+040 | SRL 2 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 6+050 | SRL 2 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 6+060 | SRL 2 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 7+920 | SRL 3 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 8+220 | SRL 3 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |
| 8+240 | SRL 3 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left |  |  |  |
| 8+570 | SRL 3 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 9+535 | SRL 3 | Hazard | 15.78 | Object Hazard Left |  | Left | Yes |  |  |
| 9+840 | SRL 3 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |
| 9+870 | SRL 3 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left |  |  |  |
| 9+935 | SRL 3 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 10+290 | SRL 3 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 11+100 | SRL 3 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 11+110 | SRL 3 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 11+120 | SRL 3 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 11+125 | SRL 3 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 12+000 | SRL 4 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 12+010 | SRL 4 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 12+480 | SRL 5 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 12+500 | SRL 5 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 12+880 | SRL 5 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |
| 12+900 | SRL 5 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left |  |  |  |
| 13+120 | SRL 5 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left | Yes |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 13+170 | SRL 5 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |
| 13+295 | SRL 5 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 13+300 | SRL 5 | Hazard | 15.78 | Object Hazard Left |  | Left | Yes |  |  |
| 13+340 | SRL 5 | Cautionary Warning Signs | 15.33 | Pedestrian Crossing |  | Left |  |  |  |
| 13+370 | SRL 5 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Left |  |  |  |
| 13+640 | SRL 5 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 13+950 | SRL 5 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 14+020 | SRL 5 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 14+030 | SRL 5 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 14+040 | SRL 5 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 15+140 | SRL 6 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 15+190 | SRL 6 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 15+500 | SRL 6 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left | Yes |  |  |
| 16+120 | SRL 6 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 16+150 | SRL 6 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 16+160 | SRL 6 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 16+170 | SRL 6 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 18+090 | SRL 7 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 18+785 | SRL 7 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 18+820 | SRL 7 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 18+860 | SRL 7 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 18+870 | SRL 7 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 18+880 | SRL 7 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 19+700 | SRL 8 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 20+240 | SRL 8 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 20+250 | SRL 8 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 20+315 | SRL 8 | Prohibitory Mandatory Signs | 14.35 | Height Limit |  | Right |  |  |  |
| 20+390 | SRL 8 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 20+390 | SRL 8 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 20+520 | SRL 8 | Cautionary Warning Signs | 15.32 | Gap In Median |  | Right |  |  |  |
| 20+520 | SRL 8 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 20+550 | SRL 8 | Cautionary Warning Signs | 15.73 | U-Turn Ahead |  | Right |  |  |  |
| 20+625 | SRL 8 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 20+625 | SRL 8 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Right |  |  |  |
| 20+730 | SRL 8 | Cautionary Warning Signs | 15.32 | Gap In Median |  | Right |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 20+750 | SRL 8 | Cautionary Warning Signs | 15.73 | U-Turn Ahead |  | Right |  |  |  |
| 20+830 | SRL 8 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 20+830 | SRL 8 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Right |  |  |  |
| 20+900 | SRL 8 | Cautionary Warning Signs | 15.32 | Gap In Median |  | Right |  |  |  |
| 20+900 | SRL 8 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 20+920 | SRL 8 | Cautionary Warning Signs | 15.73 | U-Turn Ahead |  | Right |  |  |  |
| 20+985 | SRL 8 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 20+985 | SRL 8 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Right |  |  |  |
| 21+330 | SRL 8 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 22+500 | SRL 8 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 22+550 | SRL 8 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 22+560 | SRL 8 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 22+590 | SRL 8 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 26+920 | SRL 10 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 26+930 | SRL 10 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 27+100 | SRL 10 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 27+450 | SRL 10 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 27+460 | SRL 10 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 27+470 | SRL 10 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 28+965 | SRL 11 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 29+000 | SRL 11 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 29+030 | SRL 11 | Cautionary Warning Signs | 15.02 | Right Hand Curve |  | Left |  |  |  |
| 29+400 | SRL 11 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 29+555 | SRL 11 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 30+045 | SRL 11 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left | Yes |  |  |
| 30+145 | SRL 11 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 30+210 | SRL 11 | Cautionary Warning Signs | 15.01 | Left Hand Curve |  | Left |  |  |  |
| 30+480 | SRL 11 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 30+580 | SRL 11 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 30+590 | SRL 11 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 30+600 | SRL 11 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 35+100 | SRL 12 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 35+610 | SRL 12 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 35+910 | SRL 12 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 35+920 | SRL 12 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 35+930 | SRL 12 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 37+580 | SRL 13 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Left |  |  |  |
| 37+800 | SRL 13 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 37+810 | SRL 13 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 37+820 | SRL 13 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 39+070 | SRL 14 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 39+110 | SRL 14 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRL(Service Road LHS) |  |  |  |
| Chainage | SRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 39+770 | SRL 14 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Left |  |  |  |
| 40+010 | SRL 14 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 40+510 | SRL 14 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 40+765 | SRL 14 | Cautionary Warning Signs | 15.33 | Pedestrian Crossing |  | Left | Yes |  |  |
| 40+780 | SRL 14 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 41+100 | SRL 14 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Left |  |  |  |
| 41+290 | SRL 14 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 41+300 | SRL 14 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 41+310 | SRL 14 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 40+800 | SRR 1 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 40+590 | SRR 1 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Left |  |  |  |
| 40+460 | SRR 1 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left | Yes |  |  |
| 40+450 | SRR 1 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 40+250 | SRR 1 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Left |  |  |  |
| 40+170 | SRR 1 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 39+660 | SRR 1 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 38+630 | SRR 1 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 38+600 | SRR 1 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 38+590 | SRR 1 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 38+580 | SRR 1 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 35+440 | SRR 2 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 35+420 | SRR 2 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 35+340 | SRR 2 | Prohibitory Mandatory Signs | 14.53 | Minimum Speed Limit |  | Left |  |  |  |
| 35+300 | SRR 2 | Informator y Signs | 17.35 | Bus Stop |  | Left |  |  |  |
| 35+280 | SRR 2 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left | Yes |  |  |
| 35+250 | SRR 2 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 35+130 | SRR 2 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Left |  |  |  |
| 34+650 | SRR 2 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 34+610 | SRR 2 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 34+600 | SRR 2 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 34+590 | SRR 2 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 30+125 | SRR 3 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 30+100 | SRR 3 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 29+830 | SRR 3 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Left |  |  |  |
| 29+800 | SRR 3 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 29+710 | SRR 3 | Prohibitory Mandatory Signs | 14.52 | Pass Either Side |  | Left |  |  |  |
| 29+030 | SRR 3 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 28+990 | SRR 3 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 28+800 | SRR 3 | Cautionary Warning Signs | 15.01 | Left Hand Curve |  | Right |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 28+520 | SRR 3 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 28+480 | SRR 3 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 28+470 | SRR 3 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 28+460 | SRR 3 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 26+970 | SRR 4 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 26+950 | SRR 4 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 26+750 | SRR 4 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 26+470 | SRR 4 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 26+440 | SRR 4 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 26+430 | SRR 4 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 26+420 | SRR 4 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 24+840 | SRR 5 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 24+820 | SRR 5 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 24+130 | SRR 5 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left | Yes |  |  |
| 23+980 | SRR 5 | Prohibitory Mandatory Signs | 14.31 | No Parking |  | Left |  |  |  |
| 23+630 | SRR 5 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 23+010 | SRR 5 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 22+970 | SRR 5 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 22+960 | SRR 5 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 22+950 | SRR 5 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 22+075 | SRR 6 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 22+060 | SRR 6 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 21+310 | SRR 6 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 21+100 | SRR 6 | Informator y Signs | 17.35 | Bus Stop |  | Left |  |  |  |
| 21+080 | SRR 6 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 20+770 | SRR 6 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 20+560 | SRR 6 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 20+560 | SRR 6 | Cautionary Warning Signs | 15.32 | Gap In Median |  | Right |  |  |  |
| 20+530 | SRR 6 | Cautionary Warning Signs | 15.73 | U-Turn Ahead |  | Right |  |  |  |
| 20+455 | SRR 6 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 20+455 | SRR 6 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Right |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 20+350 | SRR 6 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |
| 20+360 | SRR 6 | Cautionary Warning Signs | 15.32 | Gap In Median |  | Right |  |  |  |
| 20+340 | SRR 6 | Cautionary Warning Signs | 15.73 | U-Turn Ahead |  | Right |  |  |  |
| 20+285 | SRR 6 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 20+285 | SRR 6 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Right |  |  |  |
| 20+170 | SRR 6 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 20+170 | SRR 6 | Cautionary Warning Signs | 15.32 | Gap In Median |  | Right |  |  |  |
| 20+140 | SRR 6 | Cautionary Warning Signs | 15.73 | U-Turn Ahead |  | Right |  |  |  |
| 20+090 | SRR 6 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 20+090 | SRR 6 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Right |  |  |  |
| 19+920 | SRR 6 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 19+910 | SRR 6 | Cautionary Warning Signs | 15.32 | Gap In Median |  | Right |  |  |  |
| 19+890 | SRR 6 | Prohibitory Mandatory Signs | 14.35 | Height Limit |  | Left | Yes |  |  |
| 19+865 | SRR 6 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 19+865 | SRR 6 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 19+835 | SRR 6 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 19+835 | SRR 6 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 19+795 | SRR 6 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 19+795 | SRR 6 | Prohibitory Mandatory Signs | 14.50 | Compulsory Keep Left |  | Right |  |  |  |
| 19+520 | SRR 6 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 19+180 | SRR 6 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 19+170 | SRR 6 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 19+160 | SRR 6 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 18+375 | SRR 7 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 18+350 | SRR 7 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 18+050 | SRR 7 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |
| 17+620 | SRR 7 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 17+590 | SRR 7 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 17+580 | SRR 7 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 17+570 | SRR 7 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 15+680 | SRR 8 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 15+650 | SRR 8 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 15+180 | SRR 8 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 15+150 | SRR 8 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 14+730 | SRR 8 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 14+690 | SRR 8 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 14+680 | SRR 8 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 14+670 | SRR 8 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 13+535 | SRR 9 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 13+510 | SRR 9 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 12+970 | SRR 9 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left | Yes |  |  |
| 12+720 | SRR 9 | Hazard | 15.78 | Object Hazard Left |  | Left | Yes |  |  |
| 12+570 | SRR 9 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 12+500 | SRR 9 | Cautionary Warning Signs | 15.10 | Side Road Left |  | Left |  |  |  |
| 12+090 | SRR 9 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 12+030 | SRR 9 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 12+020 | SRR 9 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 12+010 | SRR 9 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 11+710 | SRR 10 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 11+620 | SRR 10 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 10+680 | SRR 11 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 10+650 | SRR 11 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 10+600 | SRL 11 | Cautionary Warning Signs | 15.02 | Right Hand Curve |  | Left |  |  |  |
| 10+180 | SRR 11 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 10+100 | SRR 11 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 9+580 | SRR 11 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 9+560 | SRR 11 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |
| 9+080 | SRR 11 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 7+900 | SRR 11 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 7+565 | SRR 11 | Hazard | 15.78 | Object Hazard Left |  | Left |  |  |  |
| 7+500 | SRR 11 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 7+430 | SRR 11 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 7+420 | SRR 11 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 7+410 | SRR 11 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |
| 5+570 | SRR 12 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 5+120 | SRR 12 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left |  |  |  |
| 5+100 | SRR 12 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 4+610 | SRR 12 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 4+600 | SRR 12 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 4+480 | SRR 13 | Cautionary Warning Signs | 15.49 | Speed Breaker |  | Left |  |  |  |
| 4+460 | SRR 13 | Cautionary Warning Signs | 15.36 | Two Way Operation |  | Left |  |  |  |
| 4+030 | SRR 13 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left |  |  |  |
| 4+020 | SRR 13 | Cautionary Warning Signs | 15.72 | Crash Prone Area |  | Left |  |  |  |
| 3+120 | SRR 13 | Prohibitory Mandatory Signs | 14.24 | Overtaking Prohibited |  | Left |  |  |  |
| 3+080 | SRR 13 | Cautionary Warning Signs | 15.22 | Side Road Merging From Left |  | Left |  |  |  |
| 3+000 | SRR 13 | Cautionary Warning Signs | 15.09 | Side Road Right |  | Left |  |  |  |
| 2+900 | SRR 13 | Cautionary Warning Signs | 15.02 | Right Hand Curve |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | SRR(Service Road RHS) |  |  |  |
| Chainage | SRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 2+380 | SRR 13 | Cautionary Warning Signs | 15.01 | Left Hand Curve |  | Right |  |  |  |
| 2+320 | SRR 13 | Cautionary Warning Signs | 15.10 | Side Road Left |  | Left |  |  |  |
| 2+240 | SRR 13 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 2+070 | SRR 13 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 2+060 | SRR 13 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 1+515 | SRR 13 | Hazard | 15.78 | Object Hazard Left |  | Left | Yes |  |  |
| 1+380 | SRR 13 | Hazard | 15.79 | Object Hazard Right |  | Right |  |  |  |
| 1+270 | SRR 13 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 1+260 | SRR 13 | Prohibitory Mandatory Signs | 14.01 | Stop |  | Left |  |  |  |
| 1+250 | SRR 13 | Prohibitory Mandatory Signs | 14.42 | Restriction Ends |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | IRL(Intersection Road LHS) |  |  |  |
| Chainage | IRL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 15+622 | IRL 1 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 18+489 | IRL 2 | Cautionary Warning Signs | 15.14 | Cross Road Ahead |  | Left | Yes |  |  |
| 18+489 | IRL 2 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 24+525 | IRL 3 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |
| 40+612 | IRL 4 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | IRR(Intersection Road RHS) |  |  |  |
| Chainage | IRR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |
| 17+936 | IRR 1 | Prohibitory Mandatory Signs | 14.02 | Give Way |  | Left |  |  |  |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: |  | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: |  | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: |  | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : |  | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : |  | TL(T - Junction LHS) |  |  |  |
| Chainage | TL | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: |  | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: |  | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: |  | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : |  | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : |  | TR(T - Junction RHS) |  |  |  |
| Chainage | TR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |  |  |  |  |
| Road Safety Auditor Name: |  |  | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |  |  |
| Project Name: |  |  | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |  |  |
| State/RO: |  |  | Telangana |  | PIU: | Mancherial |  |  |  |
| Start Chainage : |  |  | 0 m |  | End Chainage : | 42000 m |  |  |  |
| Length (Km) : |  |  | 42 kms |  | Carriageway : | CR(Cross - Section Road) |  |  |  |
| Chainage | CR | Type of sign board | Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Discription |  | Left/Right/Overh ead | Damaged (Yes/No) | Snapshot Image | Remarks |

| ROAD SAFETY AUDITOR RECOMMENDATION FOR ROAD SIGNAGES DURING OPERATION AND MAINTENANCE STAGE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| IRC:67-2022 (Code of Practice for Road Signs (Fourth Revision)) |  |  |  |  |  |
| RSA Name: | Chilla Suresh |  | RSA Reg No: | CRRI/TPE/2/CNP-2941/2024/02 |  |
| Project Name: | Four laning of Mancherial to Repallewada section of NH-363 (Design 0Km -42Km) |  | UPC: | N/08002/04001/TS |  |
| State/RO: | Telangana |  | PIU: | Mancherial |  |
| Start Chainage : | 0 m |  | End Chainage : | 42000 m |  |
| Length (Km) : | 42 kms |  |  |  |  |
| Summary of Additional Signages |  |  |  |  |  |
| Signage ID(Fig No.) Codal Reference IRC:67 - 2022 | Signage Decription | No. of Signs |  |  |  |
|  |  | Avenue/Left | Median/Right | Overhead | Total Signages |

# Annexure D

Chainage Wise Gap Analysis

### ANX_D ###